Revised November 25, 2018

SOME OBSERVATIONS CONCERNING RAINDROPS FALLING VERTICALLY AT THEIR EQUILIBRIUM VELOCITY

by Paul R. Paslay, P. E. #44278 (inactive)

PROLOGUE

This work presents several analyses of varying complexity evaluating the influence on the predicted shape and falling velocity of raindrops owing to assumed changes of its mathematical model.  The objective is to provide guidance in constructing mathematical models more sophisticated than those presented here.  The more sophisticated models may require the development of rather complex digital computer programs.  The choice of physical features for the sophisticated models depends on the desired prediction accuracy of their results.  In general, the sophisticated models should, from an engineering viewpoint, be as simple as possible and acceptably accurate.

The most elementary mathematical model presented here assumes the following:

All air and water properties are constant

Air/water surface tension force/length is constant

Non-uniform water flow within the raindrop is negligible (water moves as rigid body)

Shape of raindrop is spherical

Pressure differential across raindrop surface is constant

The falling velocity does not vary with time

Beginning with this primitive model, Section II shows its predictions are of limited practical value..  The relation between the pressure differential and the diameter of the raindrop is obtained.  Common “extensions” of this model are reviewed that are approximations based on this spherical model.

In Section II an example of an approximation using the model described above uses the drag on a falling, rigid sphere known from extensive prior measurements.  This drag force is a function of the falling velocity and the sphere diameter.  In this approximation the drag force must equal the buoyed weight of the raindrop.  When the raindrop diameter is specified the buoyed weight and the drag force may be found and equated to find an approximation for the falling velocity.  This approximation compares favorably with measured values of raindrop falling velocities.

Other models are considered in this report that account for the fact that the actual shape of a raindrop is not spherical, see the figure below.  Anything that makes the raindrop surface pressure differential non-uniform will cause a distortion from the spherical shape.  Section III describes an approximation to the differential pressure on the surface of the raindrop.  Two influences that cause this distortion are considered in this report.  The first is the hydrostatic water pressure differential variation with height.  The second influence is the air pressure variation on the raindrop surface caused by the falling velocity.

The water hydrostatic pressure influence is easily found while determining the raindrop surface external pressure owing to the viscous air flow over a rigid sphere is a standard fluid mechanics problem, the raindrop poses a considerably more difficult problem.  The shear stress imposed by the air on the raindrop surface will induce water flow within the raindrop.  This influence can be analyzed.  The analysis required to predict this internal water flow is complex and is not pursued in this report.  Instead, the solution for the rigid body water motion is assumed valid.  With this assumption the classical results from hydraulics may be used.

For very low falling velocities the raindrop shape can be approximated as spherical.  The Stokes viscous flow (or the Oseen approximation) solution provide an adequate approximation for these low falling velocities.  For somewhat higher falling velocities the potential flow (i. e. – zero viscosity flow) solution forms an acceptable solution in a region at the lower part of the raindrop.  The remainder of the raindrop surface is in contact with a turbulent flow region of air.  The meridian angle for the transition from potential flow to turbulent flow is called the separation angle.  Full potential flow corresponds to a separation angle of zero while full turbulent flow has a separation angle of .  The raindrop external pressure is from the potential flow solution for the bottom section.  For the top, turbulent flow, section the actual pressure is not steady but, as an approximation, its value is set equal to the stagnation pressure in this report.  Consequently, the raindrop external pressure may be determined at every point when the separation angle is known.  In this report, Section III, the separation angle is found as the angle that makes the drag calculated from the approximate pressure distribution equal to the readily available measured drag.

The results from the procedure described in the paragraph above are reported in Section IV showing the influence of the variation of the internal water pressure with and without the variable air pressure distribution.  The results for the raindrop shape show that the variable air pressure distribution is more influential and that both distributions are required for acceptable accuracy.  The inclusion of the internal water pressure distribution alone gives predictions for the raindrop shape change that are not in agreement with measured observations.

The final analysis in this report, Section V, gives an estimate of the fraction of the total drag that results from the external shear stress on the raindrop.  The calculation of this estimate is relatively involved and uses one of the more advanced techniques of fluid mechanics theory.  Boundary layer theory was pioneered by Ludwig Prandtl and it is described in detail by Hermann Schlichting in his treatise, Reference 8.  The derivation of the governing equations for this theory is based on the observation that for certain types of flows (e. g. flow over wing airfoils) the major flow disturbance from a uniform flow is near the surface of the object.  The shear stresses causing the drag on the object are found to be in a relatively thin layer on the exterior surface of the object.  Essentially, the boundary layer equations are derived by an order of magnitude study that retains only the most important terms in the general, governing equations for linear, viscous fluids.  These equations are introduced without a derivation in this Section V.  For their derivation the reader is directed to Reference 8.

The author hopes the results of the individual analyses described above will help in the further development of more sophisticated mathematical models for raindrop shapes and falling velocities.

I  INTRODUCTION AND SUMMARY

Results from analytical investigations of the shape and falling velocity of raindrops show that “nearly spherical” drops have been studied extensively.  In Battan’s, Cloud Physics, Reference 1, a meteorological description for raindrops is presented.  This book was published in 1962 and it reviews the status at that time of understanding the development of raindrops from clouds.  In particular the transition from cloud droplets (diameters in the range of 0.1 to 1.0 microns) to raindrops (diameters in the range of 500 to 5000 microns) is discussed.  Obviously, the air/water surface tension plays a role in the development of a raindrop.  In Isenberg’s, The Science of Soap Films and Soap Bubbles, Reference 2, the physics of films is presented.  The above sketch illustrates the observed alterations in shape of a falling raindrop as its volume increases.

This report presents several analyzes of mathematical raindrop models of successively increasing complexity   Departures from a spherical shape are addressed in Sections IV and V.  Each model uses constant values for surface tension as well as for air and water physical properties.  Some models include the variation in pressure (owing to gravity) within the raindrop.  In each model, the free stream velocity, U∞ or UINF, is uniform, laminar flow about the stationary raindrop. In every case the raindrop is modeled as a surface of revolution with a vertical axis.  The most complex model features considered are conventional laminar boundary layer influences on a nearly spherical shape and the assumption of a region of constant stagnation pressure above the boundary layer separation latitude.

The most elementary model considers a rigid spherical shape for the raindrop.  In order for this model to be valid the pressure differential must be uniform over the raindrop surface and surface shear stresses must vanish.  In spite of this oversimplification there are several valuable solutions using the spherical model for flowing solutions available in the literature.  A few of these solutions are reviewed in next section of this report.

Some illustrative results for successive models are obtained using rather primitive numerical integration techniques.  The illustrative results may be helpful for making judicious assumptions in future mathematical models of raindrop mechanics.

Two other important earlier publications on this subject are,

G. B. Foote and P. S. du Toit, Terminal Velocity of Raindrops Aloft, J. Applied Meteorology, vol. 8, April 1969, pp 240-253.

R. Gunn and G. D. Kinzer, The Terminal Velocity of Fall for Water Droplets in Stagnant Air, J. Meteorology, vol. 6, 1949, pp 243-248.

The second of these references gives measured results that are used for many applications.  The first reference concerns the predictions of changes from the Gunn and Kinzer work associated with changes in atmospheric conditions and includes several empirical equations deduced from the measured data.

The author feels that the following sections of this report support the results described above and in Section VI, the Conclusions.  The calculations presented here are for spherical or “nearly spherical” raindrops.  The observed severe flattening of raindrops and the subsequent breaking up of the raindrop into many parts is not considered in these analyses.  In addition, no flow within the raindrop is taken into account.  Brief reviews of each of the four principal sections are given immediately below.

II  RIGID SPHERE MODEL

The rigid sphere model allows many simplifications that make possible numerous tractable approximate solutions for the model’s behavior.  Many of these analyses yield useful estimates for the falling velocity as a function of size.  The drag may be determined using well established results.  The drag force is known to be a function of Reynolds’ number.

III  FINDING AN APPROXIMATION FOR THE PRESSURE DISTRIBUTION ON A FALLING SPHERE BASED ON ITS DRAG COEFFICIENT

The shape of a raindrop is dependent on the pressure differential distribution surrounding the drop.  A procedure is presented to estimate this pressure distribution using the classical, measured drag calculations.  The development of the estimate relies on the following three assumptions:

- The classical drag results giving the drag coefficient as a function of Reynolds’ number for a sphere.

- Continuity of the air pressure across the boundary layer separation point.

- The raindrop surface pressure above the separation is constant.

IV  NONSPHERICAL MODEL, WITH GRAVITY AND TWO AIR SURFACE PRESSURE DISTRIBUTIONS

Equations are derived for a nonspherical raindrop model that is the basis for two digital computer programs.  The shapes of the raindrop are found by numerically integrating the Young-Laplace equation.  For the first program the raindrop external pressure distribution is considered uniform.  For the second program the raindrop external pressure from the preceding section (Section III) is used in the numerical integration.  The hydrostatic variation of the pressure inside the raindrop is taken into account for both programs.   Some results from the computer programs are given to illustrate the predicted deviation from a spherical shape and the influence of the assumed external pressure distribution on the deviation from spheroidal.

V  CONSIDERATIONS INCLUDING LAMINAR BOUNDARY LAYER THEORY

In order to estimate the magnitude of the air shear stresses on the raindrop surface and their influence on the drag force, classical boundary layer theory was employed.  A solution is available in Reference 8 that is close to the problem at hand.  The Reference 8 solution was modified to obtain an estimate for the air shear stresses.  The boundary condition at the raindrop surface in this solution does not account for localized flow within the raindrop and probably yields shear stress predictions that are greater than a proper boundary condition solution would yield.  The vertical force from the shear stresses is integrated over the surface to find a force to compare to the experimentally determined total drag.

II  RIGID SPHERE MODEL

The results presented in this section are all based on the raindrop shape being spherical except fo the figure at the bottom of Page 13.  The approximation of a spherical shape for the raindrop surface greatly simplifies the mathematical analyses.  Clearly, the flow around the raindrop surface causes a surface shear stress to develop if the air viscosity is finite.  To insure the absence of this shear stress the flow must be inviscid and this is achieved by assuming potential flow of the air.  In this case, see Reference 2, the governing equation for all points on the raindrop surface layer is,

1R1 + 1R2 = Pσ

where R1 and R2 are the surface principal radii of curvature, P is the pressure differential (P = PO , a constant, in this spherical model), across the raindrop surface and  is the constant water/air surface tension.  The above equation is frequently referred to as the Young-Laplace equation.

For this spherical model the two principal radii of curvature are everywhere equal and are the radius of the spherical surface, RO.  Therefore, from the above equation,

RO = 2⋅σPO

Predictions of this equation are shown graphically below.

This relation between RO and PO is, rigorously, the sole result for this model.  However, approximations are frequently used with predictions of this model and some of these approximations are reviewed below. For example, RO can be used to estimate the drag force, D, for a rigid, falling sphere and then an estimate of the falling velocity may be found as demonstrated below.

For creeping flow on a rigid sphere the Stokes flow law is,

D = 6⋅π⋅μ⋅U∞⋅RO

or Oseen’s approximation for very low Reynolds’ Numbers may be used,

D = 6⋅π⋅μ⋅U∞⋅RO⋅(1+38⋅Re)

with Re being the Reynolds’ number.  The above two equations for drag, D, are derived based on a flow model including finite surface shear stress, see Lamb’s, Hydrodynamics, Reference 3.  The use of these equations for a sphere is an approximation.

When the drag is controlled by momentum changes (sometimes called pressure changes), an empirical form for the rigid sphere drag force is often used see Hoerner’s Fluid-Dynamic Drag, Reference 4 , e.g.,

D = CD⋅12⋅ρ⋅U∞2⋅π⋅RO2:     if Re = 1,000 to 100,000. CD = 0.47

If Re > 300,000.  CD = 0.20

etc.

The above equation is frequently included in undergraduate fluid mechanics textbooks.  It is a rough matching of drag force data measured on rigid spheres subjected to a far field uniform flow.  No account is taken of the complex surface condition that the surface shear stress induces flow within the raindrop.

Experimental results for the drag on rigid spheres have been obtained in many studies and a compilation of the results are shown in some fluid mechanics textbooks.  The figure below is taken from Reference 4, p. 3-8. The inclined, straight line at the left end of the plot is from the Oseen solution while the value of  CD is close to 0.47 in the region RE = 1,000 – 300,000.

The above plot may be partially represented up to RE = 100,000 by the following tabular form which is used in this report.

Re			CD

10		            4.0

20			2.8

- 1.6

- 1.3

- 1.2

100			1.1

200			0.8

400			0.6

600			0.5

800			0.47

1000			0.45

2000			0.42

4000			0.47

>4000&<100,000	0.47

This tabulation yields the following plot.

The conventional description for the formation of raindrops is that they originate from cloud particles that are quite small.  These particles coalesce while falling thus increasing their size, falling velocities and Reynolds’ number of the droplets.  This physical description suggests that the size of the raindrop increases monotonically with time.  Assuming this is so in our region of interest; the above curve showing measured data predicts there will be a sudden decrease in CD when the Reynolds’ number reaches about 300,000.  The following analyses show that the predicted Reynolds’ numbers for raindrops are always very much less than 300.000.

Evaluation of the approximation for the drag force, D, enables determination of an estimate for the falling velocity, VTERMINAL, using,

raindrop weight = (raindrop volume)γ = 43⋅π⋅RO3⋅γ =D

to obtain,

For Stokes flow:		VTERMINAL = 29⋅RO2⋅γμ

For Oseen’s approximation:	VTERMINAL = 29⋅RO2⋅γμ⋅11+3⋅8⋅Re

Note that Re is linear in VTERMINAL and the above equation

is therefore quadratic in VTERMINAL

Momentum controlled		VTERMINAL = 83⋅RO⋅γCD⋅ρ

The results in this section have introduced the parameters for surface tension force, , differential weight density for water and air, , the mass density for air, , and the absolute viscosity of air, .  Since this work is focused on raindrops, all illustrative results in this report use values close to the following typical parameter values.

	= 0.0004168 lbf/in

	= 0.03611 lbf/cu.in

	= 0.1124E-6 lbf−sec2/in4

	= 0.2626E-8 lbf-sec/sq.in

	= kinematic viscosity = 0.2626E-8 / 0.1124E-6 = 0.02336  in2/sec

It is interesting to note that Hoerner (loc cit) reports the falling velocity for raindrops as six feet per second (72. in/sec).  Presumably he used the Oseen drag equation for his calculations.

One estimate of the validity of the rigid spherical model can be evaluated by reviewing a deviation from the spherical shape.  Some numerical integrations using the Young-Laplace equation suggest that the shape of the raindrop may be approximated as an ellipsoid with one of its principal axes vertical.  Research has been performed on rigid, ellipsoidal bodies.  Let the ratio of the diameter to the length of the raindrop be designated by .  Based on these results, Hoerner gives an empirical result for the momentum drag coefficient for 2000 < Re < 100,000 as,

CD|2,000 < Re < 100,000= 0.44⋅α + 0.015⋅1α + 0.015⋅α

The corresponding results from the Re and  equation immediately above are shown graphically below.

The above plot shows that CD equals 0.47 when  = 1.0 (the case of a sphere).  For other values of Re the ratio of the drag coefficient from the preceding plot to 0.47 may be used to “correct” CD.  This procedure is an approximation for which the author knows of no experimental verification.  Numerical integration of the Young-Laplace equation presented below as well as direct observations of raindrops show that as the volume of the raindrop increases during falling, the value of  increases so that the raindrop flattens to a “pancake” shape.  Observed values of  exceed the maximum abscissa value in the above plot.  The value of is observed to increase to the point where the raindrop breaks up into many smaller drops.  This investigation does not explore the phenomenon of raindrop break up.  A photograph of a falling raindrop with  = 1.4 is shown in Plate VIII of Reference 1.

Another useful method for testing the practical validity of results based on the spherical model is obtained from an evaluation of the internal, pressure differential at the top of the raindrop.  When this pressure becomes less than zero, the nature of the shape prediction changes.  This is the point where flattening commences.  Note that, for a mythical spherical raindrop containing water,

Pressure at the top of the raindrop = PO − γ⋅2⋅RO

Now using the Young-Laplace equation for a sphere.,

RO = 2⋅σPO

and combining the last two equations to eliminate PO gives,

Pressure at the top of the raindrop = 2RO⋅σ − γ⋅RO2

In order that this top pressure is greater than zero the following condition must be satisfied,

RO < σγ

Using the values for  and  given on page 9 yields,

RO < 0.1074 in   or diameter < 0.2148 in

Direct observations of raindrops have shown that the curvature at the top of large raindrops changes sign as the raindrop volume increases so that the inequality should be considered as a possible limit for the spherical model to be plausible but not a limit on raindrop size.  The value of RO given above is, roughly, the maximum observed radii of raindrops.

A photograph of a non-spherical, falling raindrop is shown below.  Many such photographs may be found in the internet.  This photograph clearly shows the deviation from a sphere’

The following sketch, also from the internet, is an attempt to show the temporal progression of a raindrop shape.

III  AN APPROXIMATION FOR THE PRESSURE DISTRIBUTION ON A FALLING SPHERE BASED ON ITS DRAG COEFFICIENT

For the Reynolds’ number range of interest in this study (RE = 40 to 1E5), there are experimental results giving values for the drag coefficient and the corresponding pressure distributions on the surface of the sphere.  The results suggest that this pressure distribution can be separated into three regions, see Knudsen and Katz, Fluid Dynamics and Heat Transfer, Reference 5.  The first region occurs from the stagnation point at the bottom, φ = 0, to the meridian angle, φS, measured from the stagnation point.  φS is the separation point for the boundary layer. The pressure in this first region can be acceptably approximated by the potential flow pressure distribution. For the second region, for Re < 4E5,  extends from φS to 180 degrees.  The pressure distribution in the second region is not governed by laminar flow theory.  The pressure varies about a mean value and is always below the corresponding potential flow pressure.  The approximation introduced here assumes that the pressure is constant in this region and equal to the pressure at the separation point.  There is a third region that is not of interest here since it occurs for Reynolds’ numbers greater than 4E5.  The sketch below contains plots illustrating the pressure distributions for the three sections.  The points marked S indicate the separation points.  The ordinate, CP, in the sketch is the pressure divided by the stagnation pressure, PSTAG= 12⋅ρ⋅U∞2.

POTENTIAL FLOW = FIRST REGION

LAMINAR FLOW = SECOND REGION

TURBULENT FLOW = THIRD REGION

For a rigid sphere of radius RMAX the potential flow pressure distribution, PPF , is,

PPF = PSTAG⋅1 − 94⋅sin2(ϕ)

and the upward force applied by this distribution to the sphere in the region 0 ≤ φ ≤ φS is,

ϕ=0ϕ=φs2⋅π⋅PSTAG⋅RMAX2⋅1 − 94sin2(ϕ)⋅sinϕ⋅cos(ϕ)⋅dϕ        0 < φS< 12⋅π

To obtain the total drag force the above force must be added to the force applied by the uniform pressure, P|ϕ=φS acting on the top projected area of π⋅r|ϕ=φS2.  This gives,

CD = DRAG FORCEπ⋅RMAX2⋅PSTAG = 98⋅sin4φS    or    φS = arcsin489⋅CD

This last equation permits the determination of φS when CD is known.  CD is normally found from experimental results as described on page 10 above.

Finally, the approximation used here allows a prediction of the pressure distribution, p(ϕ) as,

pϕ = PSTAG⋅1 − 94⋅sin2(ϕ) 0≤ ϕ ≤ φS = PSTAG⋅1 − 94⋅sin2(φS) φS ≤ ϕ ≤ π

IV  NONSPHERICAL MODEL, WITH GRAVITY AND TWO AIR SURFACE PRESSURE DISTRIBUTIONS

The spherical model described above is attractive because of its mathematical simplicity.  Predictions from the spherical model for many practical problems are found with acceptable accuracy when compared with measured results.

In the author’s opinion, the two most severe restrictions imposed by the above spherical models on raindrops are,

- The internal water pressure varies within the raindrop.  This variation is neglected in the spherical models presented above.

- There are no explicit shear stresses taken into account on the outer surface of the raindrop included in the model presented above.  These shear stresses account for some of the drag, D, experienced by the raindrop.  The drag force equations presented in the model of the preceding section are empirical results based on experiments that include the shear stresses, (i.e. the total drag force).

One purpose of this section is to estimate the relative influences of the above restrictions.  In order to accomplish this two computer programs were prepared.  The first program includes the pressure variation of the water within the raindrop and assumes the outer surface air pressure to be uniform.  The second program differs from the first program only by changing the outside air surface pressure according to the distribution derived in the preceding section.  The difference in the shape predictions from the two programs illustrates the influence of the external pressure distribution.

The first of these models developed in the following addresses only Restriction 1.  That is, the variation of the internal pressure differential on the raindrop surface from gravity influences is taken into account.  In this case the shape of the raindrop surface cannot be spherical as the raindrop surface curvatures are dependent on the changing differential pressure with height.  The pressure differential for this model is dependent on height, z, and is PO − γ⋅z so the governing equation, the Young-Laplace Equation, for the raindrop surface is,

1R1 + 1R2 = PO − γ⋅zσ

where PO is the pressure differential at the bottom of the raindrop.  Since the surface is assumed here to be a surface of revolution with a vertical axis, the principal curvatures are given by,

1R1 = circumferential, principal curvature =  ±d zdxr = ±1 − d rdx2r

1R2 = r-z plane, principal curvature =  ±ddxcos−1d rdx = ∓11 − d rdx2⋅d2rdx2

Eliminating R1 and R2 using the above three equations with appropriate choice of signs yields,

−11 − d rdx2⋅d2 rdx2 + 1 − d rdx2r = PO − γ⋅zσ

with the relation between d rdx and d zdx being,

d zdx = 1 − d rdx2

A numerical integration of the differential equations may be performed using the above two equations with the initial conditions,

rx|x = 0 = 0

zx|x = 0 = 0

d rxdx|x = 0 = 1

The water/air weight density difference, and the water/air surface tension force/length, , are parameters governed by the environment so they may be measured and are assumed constant for the numerical integration.  The raindrop bottom water pressure differential, PO, must be chosen before the integration can proceed.

When a value for PO is chosen, the numerical integration on x is continued until one of two possible events occur.  These events are,

- The value of r(x) < 0.

- The value of r(x) does not return to zero as z becomes large.

In the first case, the value of z when r(x) vanishes is denoted zMAX and this is the height of the raindrop corresponding to PO.  The maximum value of r(x) in 0 < x < zMAX is denoted as rMAX.  In the second case, the value chosen for PO is too low to form an isolated raindrop using this model.  Consequently, there is a lower limit on the value of PO that can form a raindrop when  and  are specified.

The sketch below shows some of the parameters introduced in this section.

Note that the inclination parameter φ is defined by,

cosφ = d rxdx

A FORTRAN computer program was prepared to perform the numerical integration described above.  The subroutine for the numerical integration is rather primitive and is attached to this report as Appendix 1.  Three figures are shown below that present data obtained using the subroutine in Appendix 1.

The figure below is a plot showing the calculated profile for a typical case.  The input data is given on the figure with pertinent output data.

The next figure gives the maximum radius from the appendix, rMAX (RMAX in the appendix and plot), and the spherical model radius, RO, versus the bottom pressure differential, PO.  Clearly, the influence of the water/air density difference,, variation over the height is not appreciable for most of the pressure differential range considered.  This is significant as the predicted lower limit for forming a raindrop is about PO = 0.01 psi.  This was determined by running the subroutine for decreasing values of PO until the second event described above was reached.  This condition implies that the maximum diameter of a raindrop for the values of  and  considered in this plot is a little over 0.18 in.  This is consistent with the results given earlier for the rigid sphere model.  The reader should note that 2⋅RMAX/ZMAXin the above figure is predicted to be less than one.

The next figure shows the axial length of the raindrop, zMAX (ZMAX in the appendix and plot), and 2⋅rMAX (2*RMAX in the appendix and plots) as a function of the raindrop bottom pressure differential, PO.  The plot shows the two ordinates are almost equal for the larger values of PO so the aspect ratio of these raindrops is close to one.

The last plot for this constant external pressure model presents the results in the preceding plot in a different form.  The aspect ratio, 2⋅RMAXZMAX, for the raindrop is shown below.  The curve is shown after smoothing.  The unsmoothed curve “wobbles” a bit for small values of PO as suggested by the above plot.  The plot shows that the influence of liquid pressure variation with the vertical coordinate causes the raindrop aspect ratio to become less than the value for a sphere (= 1).

The above curve shows predictions that are contrary to measured, experimental results.  It is clear, based on physical grounds, that higher values of the bottom pressure, PO, are associated with 2⋅RMAXZMAX values close to 1.0 (i. e. – spherical raindrop) as the above curve predicts.  However, as PO is reduced the volume of the raindrop increases as does RMAX and ZMAX.  Experiments have shown that as PO decreases the value of 2⋅RMAXZMAX increases contrary to the trend shown in the above figure.

The second program, as pointed out above, is a modification of the first program where the external surface air pressure, p , is altered from,

p = PO − γ⋅z

to,

p = PSTAG⋅1 − 94⋅sin2(ϕ) 0 ≤ ϕ ≤ ϕS = PSTAG⋅1 − 94⋅sin2(ϕS) ϕS ≤ ϕ ≤ π

and  ϕS = arcsin489⋅CD

where PSTAG and CD are defined in the previous section.  The significant results from the second program are shown in the figures below.  The first figure gives the predicted  raindrop profile for a typical computer run.

The next plot shows that the aspect ratio 2⋅RMAXZMAX is greater than one.

The curve in the above figure shows that 2⋅RMAXZMAX → 1 as UINF → 0 (PO increasing) and 2⋅RMAXZMAX increases as UINF increases (PO decreases).

Comparison of the figures from each of the two computer programs shows modification of the external pressure on the raindrop has a greater influence on it’s shape than modification of the raindrop internal pressure.

The results of this section suggest that if the pressure distributions on the surface of raindrop are included to determine the shape of the raindrop then both inner and outer pressure variations on the raindrop surface should be included in the mathematical model.

The report by Brian Lim, Reference 6, contains an analysis similar to the one in this section.  The primary difference is the model for the pressure distribution.  Lim uses a dimensionless pressure distribution based on experimental measurements by Fage, Aeronautical Research Committee Report 1766, Reference 7.  The pressure distribution is shown in Figure 2 of Reference 6.

V  CONSIDERATIONS FROM LAMINAR BOUNDARY LAYER THEORY

The preceding section points out that the raindrop profile is altered by the stresses developed on the surface of the falling raindrop.  In order to investigate the role of the shear stresses on the surface of the raindrop a separate analysis is presented in this section.  That is, the momentum drag equation, that was obtained experimentally, actually includes the momentum drag and the drag from shear stresses.  Under many circumstances it is difficult to separately measure these shear stresses as their contribution to the drag force is often small compared to the momentum drag force.  It is possible to obtain an approximation to the shear stresses using boundary layer theory and thus separately obtain an estimate of their relative contribution to the total drag force.  Boundary layer theory is described in Schlichting’ s Boundary Layer Theory, Reference 8.  Chapter XI.  Section a4 of Reference 8 presents a relevant application for uniform flow past a solid body of revolution.

The review presented in this section is based on approximations given in Schlichting’s monograph, Chapter XI, Section a1.  These approximations, known as the “boundary layer equations,” are deduced from the Navier-Stokes equations for the external air flow relative velocity by making physically motivated assumptions for a thin flow region contacting and adhering to the rigid boundary of the sphere. The external flow outside the boundary layer is frequently the frictionless potential flow solution.  The governing equations are expressed in terms of a stream function, , and a solution procedure for finding flow solutions over axially symmetrical bodies is described.  The solution procedure uses power series in the distance, x, along a raindrop perimeter from the bottom of the raindrop.  Note that ϕ ≃ arcsinxRO.  Each power series in x is approximated here by truncation at its fifth power term.  The series are,

Radius to surface = r1⋅x + r3⋅x3 + r5⋅x5

Velocity at surface = u1⋅x + u3⋅x3 + u5⋅x5

The procedure given in Reference 8 is similar to a classical perturbation procedure yielding a differential equation in the perpendicular distance, y, from the raindrop surface for each term in the power series.  Note that this approximation fails to give a strictly proper boundary condition at the raindrop surface as no account is taken of relative motion in the internal liquid flow within the raindrop.  The speculation here is that the boundary layer theory condition used will lead to an upper bound prediction of the surface shear stress values.

At the air-water raindrop boundary the boundary condition is unusually complex.  The tangential velocities of the air and water must be continuous across the boundary.  The water inside the raindrop must develop an internal flow.  The condition at the boundary is that air velocities relative to the raindrop falling velocity are equal to the internal flow inside the raindrop.  This physical situation is reminiscent of the problem treated in Lamb’s Hydrodynamics, Reference 3, Section 229.  The solution is applicable to the early development of wind driven ocean waves and the water depth is assumed to be great.  For the problem at hand the force corresponding to the raindrop surface shear stresses will increase the average raindrop pressure, develop a tangential velocity and tend to increase the aspect ratio, 2⋅RMAXZMAX, shown in the last figure of the preceding section of this report.  These influences are usually neglected for sufficiently high values of PO.  For the remainder of this section the raindrop internal relative velocities are assumed to be zero and thus neglected.

The analysis below is a brief review of the equations governing laminar boundary layer flow appearing in Schlichting, Reference 8  A stream function, (x,y), is introduced that is related to the x and y air velocity components , u(x,y) and v(x,y).  The relationships are,

u = ∂ Ψ∂y

v = −1r⋅∂ Ψ⋅r∂x =−∂ Ψ∂ x − 1r⋅drdx⋅Ψ

where at every air-water boundary point along a meridian plane,

x	= coordinate along surface meridian, same as previous section, in

y(x)	= distance perpendicular to x direction, positive outward from raindrop, in

r(x)	= radial distance from raindrop axis to surface, a function of x, in

u(x,y)	= x component of air velocity in x-direction at boundary point x,y, in/sec

v(x,y)	= y component of air velocity in y-direction at boundary point x,y, in/sec

The governing equation for the stream function, (x,y), is,

∂3 Ψ∂y3 = 1ν⋅∂ Ψ∂y⋅∂2 Ψ∂x⋅∂y − ∂ Ψ∂x + 1r⋅d rdx⋅Ψ⋅∂2 Ψ∂y2 − U⋅d Udx

where,

	= kinematic viscosity for air, sq.in/sec

U	= U(x)	= tangential air velocity at the boundary layer outer surface of the

raindrop, in/sec

The boundary conditions for  in this governing equation are,

For y = 0 and all values of x (the raindrop surface):

 = 0 and ∂ Ψ∂y = 0

For y = ∞ and all values of x (the far field):

∂ Ψ∂y = U

The following change of variable is used to replace y by .

η = y⋅2⋅u1ν

The following two, three term power series in x are introduced into the governing equation and coefficients of each power of x are set to zero.

rx = r1⋅x + r3⋅x3 + r5⋅x5

Ux = u1⋅x + u3⋅x3 + u5⋅x5

so that the functions , f1η, f3η and f5η are defined by,

Ψx,y = ν2⋅u1⋅u1⋅x⋅f1η + 2⋅u3⋅x3⋅f3η + u5⋅x5⋅f5η

where,

η = y⋅2⋅u1ν , ,y = ,η⋅2⋅u1ν , ,η = ,y⋅ν2⋅u1

and ,ξ implies the partial derivative of  with respect to .  The individual terms in the approximating governing equation,

ν⋅Ψ,yyy = Ψ,y⋅Ψ,xy − Ψ,x + r,xr⋅Ψ⋅Ψ,yy − U⋅U,x

become, through fifth power in x and with a prime designating the derivative with respect to the argument ,

ν⋅Ψ,yyy = 2⋅u12⋅x⋅f1'''η + 4⋅u1⋅u3⋅x3⋅f3'''η + 2⋅u1⋅u5⋅x5⋅f5'''η

Ψ,y⋅Ψ,xy = u12⋅x⋅f1'η2 + 8⋅u1⋅u3⋅x3⋅f1'η⋅f3'η + 6⋅x5⋅u1⋅u5⋅f1'η⋅f5'η + 2⋅u32⋅f3'η2

Ψ,x⋅Ψ,yy = u12⋅x⋅f1η⋅f1''η + u1⋅u3⋅x3⋅2⋅f1η⋅f3''η + 6⋅f1''η⋅f3η + x5⋅u1⋅u5⋅f1η⋅f5''η + 5⋅f1''η ⋅f5η + 12⋅u32⋅f3η⋅f3''η

r,xr = 1x + 2⋅r3r1⋅x + 2⋅2⋅r5r1 − r32r12⋅x3 − 2⋅3⋅r3⋅r5r12 − r33r13⋅x5

r,xr⋅Ψ⋅Ψ,yy = u12⋅x⋅f1η⋅f1''η + 2⋅x3⋅u1⋅u3⋅f1η⋅f3''η + f1''η⋅f3η

U⋅U,x = x⋅u12+x3⋅4⋅u1⋅u3+x5⋅6⋅u1⋅u5+3⋅u32

while for the special case of a sphere of radius Rs with a far-field uniform velocity of U∞, the values for u1, u3 and u5 as well as r1, r3 and r5 are determined from,

U⋅U,x = 94⋅U∞2Rs2⋅x − 32⋅U∞2Rs4⋅x3 + 310⋅U∞2Rs6⋅x5

rx = x − 16⋅Rs2⋅x3 + 1120⋅Rs4⋅x5

When the above five equations representing boundary layer theory are substituted into the governing equation for , the resulting equation can be formulated as the sum of three terms.  These terms are proportional to x, x3 andx5. Since, for this approximation, the equation must be valid for all values of x, the multipliers of x, x3 and x5 must separately vanish.  Consequently, the governing equations for this approximation with U(x) defined by u1, u3 and u5 and r(x) defined by r1, r2 and r3 requires the following equations be satisfied.

f1'''η = −f1η⋅f1''η + 0.5⋅f1'η2 − 1

f3'''η = −f1η⋅f3''η + 2⋅f1'η⋅f3'η −2⋅f1''η⋅f3η − 0.5⋅r3r1⋅u1u3⋅f1η⋅f1''η − 1

f5'''η = −f1η⋅f5''η + 3⋅f1'η⋅f5'η − 3⋅f1''η⋅f5η − 3               +2⋅u32u1⋅u5⋅3⋅f3'(η2−4⋅f3η⋅f3''η − 0.75  −2⋅r3r1⋅u3u5⋅f1η⋅f3''η+f1''η⋅f3η − 2⋅r5r1 − r32r12⋅u1u5⋅f1η⋅f1''η

These three equations were derived following the procedure in Schlichting’s Reference 8 and an illustrative example of a sphere is given there.  The derivation assumed that the coefficients r1, r3 and r5 as well as u1, u3 and u5are known as the profile of the axially symmetric body and the potential flow solution are assumed known.  For the case of the raindrop the profile must be determined as part of the solution to this problem and the potential flow solution for the configuration must be found.  The formulation must be altered to replace the coefficients r1, r3 and r5 as well as u1, u3 and u5.  Since these coefficients are not known a’priori a solution procedure would be rather complex.

The author feels that results for potential flow over a sphere, in view of the mathematical complications of a non-spherical object, is adequate to find an estimate of the drag from surface shear stresses compared to the total measured drag.  The boundary condition for y = ∞ (or  = ∞) is replaced with  = 5.   This limit on  = y⋅2⋅u1ν determines the thickness of the boundary layer and is consistent with the values used in the analyses in Reference 8.

A digital computer program was prepared that approximates the air velocity using the potential velocity distribution for a sphere whose size is determined using PO.  When the shape is found the three terms for r(x) are determined.  The governing equations for f1, f2 and f3 are then integrated numerically.  Obviously, they must be solved sequentially.

Plots for f1η, f3η and f5η and their first derivatives with respect to  are shown graphically below.  These results were obtained by numerical integration of the above differential equations.

The above plot suggests that the limit for  should be greater than 5 in order to improve accuracy of the solution.  This suggestion is not pursued here.

The raindrop surface shear stress may be calculated from the functions f1η, f3η and f5η evaluated at  = 0.  The result is shown below with the surface shear stress,  as a function of the meridian angle .

In order to estimate the contribution of this boundary layer shear stress to the total drag two elementary calculations are presented.  The first is the calculation for the total drag reviewed above using the coefficientCD.  The Reynolds’ number Re for this calculation is approximated by,

Re = ρ⋅U⋅2⋅ROμ = 0.1124E-6⋅66.7⋅2⋅0.020.27E−8 = 111.1

so that, from the plot on Page 6, CD  1.2 and,

Total Drag = CD⋅Stagnation Pressure⋅Projected Area = π2⋅CD⋅ρ⋅U2⋅RO2

= 1.5708⋅1.2⋅.1124E-6⋅66.72⋅0.022 = 0.3770E-6 lbf

The second calculation is an integration of the shear stress from the boundary layer solution over the lower part of the raindrop surface.  The vertical, upward force, FO, associated with this shear stress,  over the region  = 0 to  = O is,

FO = φ = 0φ = φO2⋅π⋅RO2⋅τφ⋅sin φ2⋅dφ

A numerical integration with O = 100.8 degrees yields the maximum value for FO of,

FO = 0.184E-6 lbf

Comparison of the total drag with the force FO shows that for a raindrop with a diameter of approximately 0.04 inches the drag from shear stress accounts for about half the total drag.

A second example for a raindrop diameter of approximately 0.2 inches yields the following results,

UO = 427. in/sec

Re = 3560

CD = 0.46

Total Drag = 140.8E-6 lbf

FO|MAX,φO = 90o= 3.22E-6 lbf

FO|SEPARATION, φ = 142.6o = 0.0855E−6 lbf

These results show that for the 0.2 diameter raindrop the drag from the shear stress is small compared to the total drag.

VI  CONCLUSIONS

A single analysis of a comprehensive, mathematical model of a raindrop would be rather complex and is not pursued in this work.  Instead, several elementary analyses are presented here in order to emphasize several important features of raindrop behavior.  Most of the results presented here are not new, as the references to the cited publications confirm.  Based on the results of the elementary analyses and the publications, the author believes the following conclusions are justified.

- The usual concept presented in the literature of cloud droplets coalescing into a raindrop is the primary phenomenon in raindrop formation is consistent with the analyses presented here.

- When the nominal raindrop diameter, DNOM, is less than 0.05 inches, The shape of the raindrop is nearly spherical so that an analyses based on a spherical model is acceptable.

- When the raindrop nominal diameter becomes greater than 0.05 inches, the raindrop shape deviates from spherical and this deviation increases with nominal diameter.  The diameter to height ratio of the raindrop increases as the nominal diameter increases.  Finally, a point is reached where a raindrop breaks apart into several smaller drops and the coalescing phenomenon becomes active on each of the smaller drops.  This work does not attempt to analyze the break-up phenomenon.

- The so-called “terminal falling velocity” of a raindrop may be estimated by equating the instantaneous drag force to the instantaneous buoyed weight of the raindrop.  This is not, strictly speaking, a terminal velocity since the raindrop velocity is changing and thus has a nontrivial acceleration.  The terminal velocity estimation is good for most engineering purposes.  This velocity estimate is about 6.0 ft/sec when the nominal diameter is 0.1 inches.  There is a jump in estimated velocity to about 14.0 ft/sec when the nominal diameter equals 0.12 inches.  This jump occurs when the drag force, governed Oseen drag for smaller diameters, changes to momentum drag for larger diameters.  Momentum drag determines the velocity when the nominal diameter is greater than 0.12 inches and this velocity increases to about 20.0 ft/sec when the diameter is 0.24 inches.

- An approximate solution using boundary layer theory is used to estimate the contribution to the drag force from the shear stresses developed by the air flow on the raindrop surface.  This analysis predicts the fractional contribution to the drag force from the shear stresses depends on the raindrop size.  It is appreciable for a 0.04 inch diameter raindrop and negligible when the diameter is 0.20 inches.

VII  REFERENCES

- Louis J. Battan, Cloud Physics, Dover Publications, Inc., Mineola, New York, 2003

- Cyril Isenberg, The Science of Soap Films and Soap Bubbles, Dover Publications, Inc., New York, 1992

- Sir Horace Lamb, Hydrodynamics, Sixth Edition, Dover Publications, New York, 1945

- Sighard F. Hoerner, Fluid-Dynamic Drag, Published by the author, 1965

- James G. Knudsen and Donald L. Katz, Fluid Dynamics and Heat Transfer, McGraw-Hill Book Company, Inc, New York, 1958

- Brian Lim, “Derivation of the Shape of Raindrops,”  http://www.cs.cmu.edu/byl/publications/raindrop.pdf or bing, the shape of raindrops

- A. Fage, “Reports and memoranda – Aeronautical Research Committee 1766, “ Vol. 20, 1937

- Hermann Schlichting, Boundary Layer Theory, Seventh Edition, McGraw-Hill Book Company, New York, 1979

VIII  PARTIAL NOMENCLATURE

CD		= drag coefficient for total drag.

CP		= pressure divided by stagnation pressure

D		= upward drag force on raindrop, lbf

DNOM		= nominal diameter  2⋅3raindrop volumeπ, in

f1, f3,f5	= functions of in boundary layer solution

FO		= upward force developed by  from boundary layer solution, lbf

P		= inner/outer pressure differential on surface of raindrop, P = P(x), psi

PO		= inner/outer pressure differential at bottom of raindrop,  = 0, psi

PAIR		= far field constant air pressure, psi

PPF		= pressure distribution from potential flow solution, psi

PSTAG		= stagnation pressure, =12⋅ρ⋅U∞2, psi

r(x)		= radial coordinate measured perpendicular to raindrop axis, r= r(z), in

r1, r3, r5	= coefficients in series for r(x)

Rs		= radius of sphere for boundary layer equation, in

Re		= Reynolds’ Number, a dimensionless number = ρ⋅U⋅2⋅ROμ

RMAX		= maximum value of r(x) for raindrop profile, in

RO		= radius of curvature at bottom of raindrop surface, in

R1		= first principal radius of curvature for raindrop surface, in

R2		= second principal radius of curvature for raindrop surface, in

u(x,y)		= boundary layer theory x component of velocity, in/sec

u1, u3, u5	= coefficients for velocity at raindrop surface, potential flow solution

U = U(x)	= tangential air velocity at the raindrop outer boundary layer, in/sec

U∞		= far-field velocity for boundary layer equation, in/sec

v(x,y)		= air velocity component in y-direction, in/sec

VTERMINAL 	= raindrop instantaneous falling velocity, in/sec

vol		= volume of raindrop = π8⋅nominal diameter3 , in3

x		= coordinate measured from bottom along raindrop surface meridian, in

y		= coordinate perpendicular to x, outward perpendicular to surface, in

z		= coordinate measured from bottom along raindrop vertical axis, in

ZMAX		= height of raindrop, in

α		= raindrop ratio of maximum diameter to length

γ		= weight density differential, water – air, lbf/cu.in.

η		= dimensionless parameter for y, y⋅2⋅u1ν

μ		= absolute viscosity of air, psi-sec

		= kinematic viscosity of air, in2/sec

ρ		= mass density of air, lbf-sec2/in4

σ		= air/water surface tension, lbf/in

		= boundary layer theory shear stress on raindrop surface, psi

or ϕ		= angle along a meridian measured from the bottom of the raindrop, rad.

φS or ϕS 	= boundary separation value for or ϕ, radians

Ψx,y	= stream function for air flow, in2/sec

IX  ACKNOWLEDGEMENTS

The author is indebted to his good friend and hydraulics tutor Professor Herbert K. Beckmann (deceased) for teaching him the technical approach used in Sections II through IV of this report.  His influences are apparent in each of these sections.

The careful reading of this manuscript and resulting comments by David Hearn and Udaya B. (UB) Sathuvalli were very helpful and appreciated.

APPENDIX 1, FORTRAN SUBROUTINE QFRA

C=======================================================================

SUBROUTINE QFRA

* (XM, YM,

* UINF, GAM, RHOAIR, NUAIR, ST,

* PI, DPHI, RO, UMAX, PZ, TRA, RA,

* R1, R2,

* PHIA, TZA, TX, DZ, DR, TZ,

* TR, TDZ, TDR, TDX,

* PHI, Z, ZA, DX, X, R, RP, ZP, POP,

* TINVR1, INVR1, TR1, TR2, RPXOR, RMAX, ZMAX, RE, CD, PHIS,

* U, UX, PSTAG, FMOM, WEIGHT, ERR, IRMAX, NPT)

C

REAL*8

* XM, YM,

* UINF, GAM, RHOAIR, NUAIR, ST,

* PI, DPHI, RO, UMAX, PZ, TRA, RA, RE,

* R1(181), R2(181), RET(14), CDRET(14),

* PHIA, TZA, TX(181), DZ(181), DR(181), TZ(181), POP(181),

* TR(181), TDZ(181), TDR(181), TDX(181),

* PHI(181), Z(181), ZA, DX(181), X(181), R(181), RP(181), ZP(181),

* TINVR1, INVR1, TR1(181), TR2(181), RPXOR(181), RMAX, ZMAX,

* U(181), UX(181), PSTAG, FMOM, WEIGHT, ERR, CD, PHIS, XXP

C

INTEGER  I, J, K, IRMAX, NPT

C

PI = 4.D0 * DATAN(1.D0)

C

UMAX = 1.5D0 * UINF

C

RO = XM

C

C     REYNOLDS' NUMBER IS BASED ON RO

C

RE = 2.D0 * UINF * RO / NUAIR

C

IF (RE .LT. 10.D0) THEN

C

PRINT *,' RE < 10, OUT OF RANGE, RE = ', RE

C

STOP

C

ELSE IF (RE .GT. 100000.D0) THEN

C

PRINT *,' RE > 100,000, OUT OF RANGE, RE = ', RE

C

STOP

C

END IF

C

C      CONVENTIONAL DRAG FOR SPHERE FOR RE = 40 TO 100,000, CDRE(Re)

C

RET(1)  = 10.D0

C

RET(2)  = 20.D0

C

RET(3)  = 40.D0

C

RET(4)  = 60.D0

C

RET(5)  = 80.D0

C

RET(6)  = 100.D0

C

RET(7)  = 200.D0

C

RET(8)  = 400.D0

C

RET(9)  = 600.D0

C

RET(10) = 800.D0

C

RET(11) = 1000.D0

C

RET(12) = 2000.D0

C

RET(13) = 4000.D0

C

RET(14) = 1.0D5

C

CDRET(1) = 4.0D0

C

CDRET(2) = 2.8D0

C

CDRET(3) = 1.6D0

C

CDRET(4) = 1.3D0

C

CDRET(5) = 1.2D0

C

CDRET(6) = 1.1D0

C

CDRET(7) = 0.8D0

C

CDRET(8) = 0.6D0

C

CDRET(9) = 0.5D0

C

CDRET(10) = 0.47D0

C

CDRET(11) = 0.45D0

C

CDRET(12) = 0.42D0

C

CDRET(13) = 0.47D0

C

CDRET(14) = 0.47D0

C

CALL LOCATE(RET,13,RE,K)

C

CD = CDRET(K) + ((CDRET(K+1)-CDRET(K))*(RE-RET(K))

*         /(RET(K+1)-RET(K)))

C

XXP = (8.D0*CD/9.D0)**0.25

C

IF (XXP .GE. 1.D0) THEN

C

PHIS = 3.14159D0/2.D0

C

ELSE

C

PHIS = DASIN(XXP)

C

END IF

C

C     INITIAL CONDITIONS

C

I = 1

C

PHI(I) = 0.D0

C

X(I) = 0.D0

C

R(I) = 0.D0

C

R1(I) = -RO

C

R2(I) = RO

C

Z(I) = 0.D0

C

ZP(I) = 0.D0

C

RP(I) = 1.D0

C

PZ =  0.5D0 * RHOAIR * (UINF**2)

C

POP(I) = PZ

C

RPXOR(I) = 1.D0

C

U(I) = 0.D0

C

UX(I) = 1.0D0

C

C     START NUMERICAL INTEGRATION

C

NPT = 181

C

DO 110 I = 2, NPT

C

PHI(I) = PHI(I-1) + DPHI

C

PHIA = 0.5D0 * (PHI(I) + PHI(I-1))

C

TDX(I) = DABS(R1(I-1)) * DPHI

C

TDR(I) = TDX(I) * DCOS(PHIA)

C

TDZ(I) = TDX(I) * DSIN(PHIA)

C

TX(I) = X(I-1) + TDX(I)

C

TR(I) = R(I-1) + TDR(I)

C

TRA = 0.5D0 * (TR(I) + R(I-1))

C

TZ(I) = Z(I-1) + TDZ(I)

C

TR2(I) = TRA / DSIN(PHIA)

C

TZA = 0.5D0 * (TZ(I) + Z(I-1))

C

IF (PHI(I) .LT. PHIS) THEN

C

POP(I) = PZ * (1.D0 - 9.0D0 * (DSIN(PHI(I))**2) / 4.D0)

*           - GAM * TZA

C

ELSE

C

POP(I) = PZ * (1.D0 - 9.0D0 * (DSIN(PHIS)**2) / 4.D0)

*           - GAM * TZA

C

END IF

C

TINVR1 = POP(I) / ST - 1.D0 / TR2(I)

C

TR1(I) = 1.D0 / TINVR1

C

DX(I) = 0.5D0 * DABS(R1(I-1) + TR1(I)) * DPHI

C

DR(I) = DX(I) * DCOS(PHIA)

C

DZ(I) = DX(I) * DSIN(PHIA)

C

X(I) = X(I-1) + DX(I)

C

R(I) = R(I-1) + DR(I)

C

IF (R(I) .GT. R(I-1)) THEN

C

RMAX = R(I)

C

IRMAX = I

C

END IF

C

Z(I) = Z(I-1) + DZ(I)

C

RA = 0.5D0 * (R(I) + R(I-1))

C

R2(I) = RA / DSIN(PHIA)

C

ZA = 0.5D0 * (Z(I) + Z(I-1))

C

U(I) = UMAX * DSIN(PHIA)

C

UX(I) = (U(I) - U(I-1)) / DX(I)

C

INVR1 = POP(I) / ST - 1.D0 / R2(I)

C

R1(I) = 1.D0 / INVR1

C

RP(I) = DCOS(PHIA)

C

RPXOR(I) = RP(I) * X(I) / R(I)

C

IF (I .EQ. 181) THEN

C

ZMAX = Z(181)

C

PSTAG = PZ

C

FMOM = CD *(1.D0/2.D0)*PI*RHOAIR*(UINF**2)*(RMAX**2)

C

WEIGHT = 0.D0

C

DO 105 J = 2, 181

C

WEIGHT = WEIGHT + 0.25D0*PI*GAM*((R(J)+R(J-1))**2)

*                     *(Z(J)-Z(J-1))

C

105   CONTINUE

C

ERR = FMOM / WEIGHT - 1.D0

C

YM = FMOM - WEIGHT

C

END IF

C

110 CONTINUE

C

120 CONTINUE

C

130 CONTINUE

C

200 CONTINUE

C

RETURN

C

END

C

C***********************************************************************