March 30, 2017

ELEMENTARY LOAD ANALYSES FOR THREADED PRODUCTION CONNECTORS IN OIL WELLS

by Paul R. Paslay, P.E. #44278, Manatee Inc., F-4992

INTRODUCTION

A discussion concerning thread design has been included in most machine design textbooks for many years (e. g. Joseph Edward Shigley, Mechanical Engineering Design, McGraw-Hill Book Company, 1963).

An early, significant study of the modeling of thread behavior was given by J. H. Goodier in 1940 (ASME, Journal of Applier Mechanics, v. 7, pp A10-A16, “Distribution of Load on Threads and Screws”).

This study includes detailed analyses for two mathematical models with different approximations for the teeth.  These models are intended to be applicable to the connectors used for production tubing in oil wells.  For these connectors, there are two overlapping tubes with teeth in the radial gap between the tubes.  The number of teeth along the length of the connector is typically about 10. A salient feature of all connectors is the axial stiffness of the teeth at their contact points relative to their base.  The geometry of these connectors differs from conventional machine design geometry assumptions in two important ways. They are,

- The walls of the overlapping tubes, which are the bases of the teeth, are sufficiently thin for rotational flexibility of the base of the teeth to be non-negligible in some cases.  Base rotation of a tooth causes an axial displacement that reduces the axial stiffness of the tooth at its contact point.  A good model should include this influence when calculating axial stiffness.

- In general, the tooth contact loads vary from tooth to tooth.  The magnitude of the contact loads is usually smaller at the center of the axial row of teeth.  On the one hand, it may be desirable to have the tooth contact loads constant.  On the other hand, since one function of the connector is to prevent leakage, a central section with sufficiently large tooth contact loads that do not cause yielding of the tooth material under extreme anticipated loads may be desirable.  Consequently, a desirable model should calculate the tooth contact load for every tooth.

The reader should note that two features of the oil well production tubing are being neglected in the models presented in this study.  They are,

- No account is taken for tightening the connection.  Tightening is usually accomplished by having shoulders on each element of the connector that contact and develop a resistance to further relative rotation.  When the contact load between the shoulders reaches a specified value, the connector is defined as being tightened.  The two computer programs for the models presented in this study can easily be modified to include this phenomenon.

- The oil well version of the connector has a small taper in its diameter in order to make field connection easier and faster.  This feature is not included in either of the models presenter here.

Contemporary studies of oil well production connectors have used finite element (or similar) techniques .  It is clear that for a specified design configuration these methods provide the most accurate results currently available.  The primary disadvantage of the finite element methods is they are not well suited to performing the many cases required for design optimization.  The time and expense involved in optimization using finite element methods suggests that a less elegant, fast and technologically simpler method of analysis may streamline the optimization process by initially narrowing ranges of the searched parameters.

This study presents two analyses to predict, approximately, the load and displacements of an oil well production tubing connector.  The two analyses are both models of the threads being between two overlapping thin walled tubes.  The first analysis is of the more complex model and it establishes approximations for three parameters used in the simpler, second analysis.  The simpler, second analysis replaces the teeth in the first analysis by an elastic layer bonded to the bounding tube surfaces.  The computer programs (in FORTRAN) for each analysis execute in less than one second running time on conventional PC computers.

One useful result from the two analyses is that an approximation for the variation in tooth loads along the connector is found.  Predictions for displacements are also found.

The author feels these analyses (or modifications of them) can be helpful as a first step to design optimization since program parameters can be quickly and easily varied over a wide range of values.

OVERVIEW OF THE TWO ANALYSES

FIRST ANALYSIS:

The mathematical model for this analysis consists of two overlapping, thin walled, elastic, right circular cylinders.  The overlap length is designated by L.  The overlapping annulus between the bounding cylinders contains equally spaced, identical teeth on alternating sides of the annulus.  The profile of the teeth resembles Acme standard threads.  An axial position coordinate, x, is defined by its distance from the axial beginning of the overlapping annulus.  The inner cylinder axial displacement is set to zero at x = 0.  The axial force on the outer cylinder at x = 0 is set to zero.  The position coordinate, x , at the end of the overlapping region is equal to L.  The axial force on the outer cylinder at x = L is set equal to F while the force on the inner cylinder at x = L is set to zero.  Each of the cylinders has a station assigned at the location of the center of the base of each tooth.  The analysis constructs the spring matrix for this mathematical model.  Local contributions to the spring matrix are determined from analysis of the model described above.  Each cylinder is treated as a thin walled cylinder using conventional shell theory.  Calculations for each tooth deformation use beam bending with shear deflection theory.  The mathematical model is thus reduced to a set of linear, algebraic equations.  The size of the coefficient matrix depends on the number of teeth, N, and is (2*N-1)X(2*N-1).  The sketch below shows the numbering scheme for the stations and for the elastic elements joining the stations for N = 5 teeth.  The patterns for other values of N can be deduced from this sketch.

Details of the first analysis may be found in Appendices 1 and 2 of this report.

SECOND ANALYSIS:

The mathematical model employed for the first analysis is altered in order to use some of the results from the first analysis and to produce up to about 15 equally spaced spatial stations along the overlapped region.  The results from the first analysis for the relative axial displacements between the two cylinders owing to the axial force applied to the outer cylinder at x = L is used to calculate an assumed constant shear stiffness for the elastic annulus between the cylinders.  The mathematical model studied for the second analysis has the same general configuration (see Page 3) except the teeth are replaced by a linear elastic shear layer filling the annulus between the cylinders.  The computer program for the first analysis gives the values for the stiffnesses to be used in the second model analysis.  The output from the second analysis gives the forces and displacements along the connector so that the entity that is being optimized can be evaluated as the model input parameters are varied.

RESULTS

The above discussion outlines two analyses that can be used to aid the optimization of chosen features of the design of oil well production tubing connectors.  The advantage of the method is that the mechanical model for the connector is simplified through assumptions that allow rapid and inexpensive approximations using a laptop digital computer rather than the more complex conventional finite element methods (FEM).

There is no doubt that the FEM is currently the preferred method of calculation for connectors when accurate predictions of forces and displacement distributions are required.

When a new connector design is being developed, there is often a specific target derivable from the output data for one of the analyses, for the design (e.g. – desired load distribution on the threads).  An accurate configuration description of a connector including tooth profiles requires a large number of independent parameters.  In order to reduce the required amount of computation, an approximate solution method can be useful for identifying parameter combinations that produce promising target results   When these are found, it is prudent to continue the optimizing search using FEM.

To demonstrate the nature of the rapid calculations an illustrative problem is presented here. Program KMAT1 requires input data that describes a vastly simplified model of a connector.  The output file from this program provides a prediction of tooth loads and displacements as well as a prediction of the spring constants used in the second analysis.   The illustrative problem is defined by the input data required for the computer program, Program KMAT1.  The data for the illustrative problem are,

FOR      	= force/(edge length) at loaded end of outer cylinder, lbf/in	= 1.0

RADT		= mean radius of outer cylinder, in				= 2.15

HWT      	= wall thickness of outer cylinder, in				= 0.1

RADB     	= mean radius of inner cylinder, in				= 2.00

HWB      	= wall thickness of inner cylinder, in				= 0.1

EP       		= Young's modulus of elasticity, psi				= 30.E6

NU       	= Poisson's ratio						= 0.3

LOT      	= outer tooth height for contact, in				= 0.025

HOT      	= outer tooth base width at cylinder, in			= 0.15

L0B      	= inner tooth height for contact, in				= 0.025

HOB      	= inner tooth base width at cylinder, in			= 0.15

LSEP		= axial separation between teeth, in				= 0.225

NT       	= number of teeth						= 20

The following figures are from data in the output file of Program KMAT1 which is a FORTRAN computer program based on the first analysis.

In addition to the displacements and forces plotted above, there are other pertinent data included in the output file from Program KMAT1.  The pertinent data are,

GEOMETRIC DETAILS

OCOR  	= outer cylinder outer radius, in       	=    2.200

OCIR  	= outer cylinder inner radius, in       	=    2.100

OCTHK 	= outer cylinder wall thickness, in     	=    .1000

ICOR  	= inner cylinder outer radius, in       	=    2.050

ICIR  	= inner cylinder inner radius, in       	=    1.950

ICTHK 	= inner cylinder wall thickness, in     	=    .1000

ELOR  	= tooth gap outer radius, in            	=    2.100

ELIR  	= tooth gap inner radius, in            	=    2.050

ELTHK 	= tooth gap thickness, in               	=    .5000E-01

LEN   	= axial length for all teeth, in        	=    4.500

SOME INTERMEDIATE VALUES FOR LOADS/(EDGE LENGTH)

DMOB  	= bottom (moment/edge=1)/(db*(betab^2))   	=    .4461E-04

DMOT  	= top (moment/edge=1)/(db*(betab^2))      		=    .4681E-04

BETAB 	= outer value of beta (see Appendix 1)                   	=    2.874

BETAT 	= inner value of beta (see Appendix 1)                      	=    2.772

CB    	= bottom tooth, lateral spring constant, psi	 	=    .5042E+07

CT    	= top tooth, lateral spring constant, psi    		=    .5149E+07

KB    	= bottom, axial spring constant, psi	   		=    .1317E+08

KT    	= top, axial spring constant, psi	      		=    .1349E+08

The Program KMAT1 may be suited to some optimization studies.  Other optimization studies, such as variation of cylinder design, can benefit by assuming the elastic behavior of the teeth can be modeled with an elastic layer in the annulus between the tubes.  This approximation is compelling when the number of teeth is large.  The second analysis models the teeth as an elastic shear layer within the annulus between the cylinders.  The next illustrative problem gives results from Program SPRTESF that is based on the second analysis.  Values for the input data are chosen to match, approximately, the case studied using Program KMAT1 given above.  The input data required to execute Program SPRTESF is smaller than Program KMAT1 but it requires values of KT, KC and KB either calculated separately or determined using the output data from Program KMAT1.  The input data for Program SPRTESF for the illustrative case is,

N	= number of x station values, (total points = 2*N-1)  	= 15

KT	= outer axial spring constant, psi 				= 13.49E6

KC	= annulus shear spring constants, psi	 			= 2.547E6

KB	= inner axial spring constants, psi 				= 13.17E6

Unlike Program KMAT1, the parameter N in the second analysis, Program SPRTESF, does not necessarily equal the number of teeth .it simply locates the equally spaced stations.  The outer and inner spring constants, KT and KB, are chosen directly from the Program KMAT1 output file.  The value for the annulus shear spring constant, KC, represents both teeth so, in this case, it is set to one-half the mean value of CT and CB.  The output file for Program SPRTESF contains values of the junction displacements and the member forces (see Page 3).  The results for this illustrative problem are shown graphically in the following two plots.

Since the above two figures from Program SPRTESF are intended for the same problem as the earlier figures from Program KMAT1, it is worthwhile to compare the results from Program SPRTESF, Figures 3 and 4, with results from Program  KMAT1, Figures 1 and 2,

Comparison of Figures 1 and 3 (Displacements)

- The axial displacements in Figure 1 (KMAT1) are greater than those of Figure 3 (SPRTESF) by about 40 %.

- In the central section of the connectors the axial displacements of the inner junctions exceed, by a small amount, the axial displacements of the outer junctions for Program SPRTESF.  These displacements are essentially equal for Program KMAT1

Comparison of Figures 2 and 4 (Forces )

- The force comparison is very favorable since the values of the forces are very close to one another.

- The “flat” central region in Figure 2 (SPRTESF) is a bit longer than the corresponding central region in Figure 4 (KMAT1)

For the above two figures the comparison of the output files from Programs KMAT1 and SPRTESF seems to be favorable.

Additional results are given below to suggest how Program SPRTESF may be used for the initial stage of design optimization.  Figures 5 and 6 give results using earlier data for Program SPRTESF except that the shear stiffness of the elastic shear layer is varied.  Figure 5 shows the displacement at the loaded end for both the outer and inner cylinders.  The relative displacements at the loaded end decrease as the shear layer stiffness increases.  The plot gives a quantitative prediction to the expected trend of the data.

Figure 6 shows how the maximum and minimum shear loads in the elastic shear layer vary as the stiffness of the shear layer is varied.  The maximum shear load may be interpreted as an approximation to the tooth load per unit axial length at the external load application point.  The minimum tooth shear load per unit length occurs near the center of the connector model in this case.  Two observations concerning Figure 6 are,

  - the maximum tooth load per unit length is at the loaded end.

  - the minimum tooth load per unit length can be much smaller than the maximum tooth load per unit length

The author hopes the results presented in this report will be helpful in the optimization of production and other types of connector designs.

APPENDIX 1, USEFUL ELASTIC, THIN-SHELL SOLUTION

SOLUTION GIVEN BY TIMOSHENKO & S. WOINOWSKY-KRIEGER

At the beginning of Chapter 15, the textbook, Theory of Plates and Shells, contains a solution for a thin walled right circular cylindrical shell.  The coordinate, x, for the axis of the shell extends from x = 0 to infinity (i.e. – a semi-infinite length shell).  At x = 0 a constant edge moment per unit circumferential edge length, MO, and a constant edge radial shear force per unit circumferential edge length, QO, are applied.  There are no other externally applied loads.  The governing equation for the inward radial displacement, w = w(x), of the cylindrical shell is derived in the textbook.  MO and QO, when positive and applied separately, both give w < 0 and d wdx > 0 at x = 0.

The governing homogenous differential equation for w is,

d4wdx4 + 4⋅β4⋅w = 0

where,

β4 = 3⋅1 − ν2a2⋅h2

and,

 = Poisson’s ratio

a = radius of cylinder middle surface

h = cylinder wall thickness

The moments M(x), and shear forces, Q(x), per unit of edge length are determined by,

M(x) = −D⋅d2 wdx2		and 		Q(x) = −D⋅d3 wdx3

where D is the plate constant given by,

D = E⋅h312⋅1 − ν2

with E = Young’s modulus and  = Poisson’s ratio.

The homogenous solution for w is given as,

w = eβ⋅x⋅C1⋅cos (β⋅x) + C2⋅sin β⋅x + e−β⋅x⋅C3⋅cos β⋅x + C4⋅sin β⋅x

where C1, C2, C3 and C4 are constants of integration determined by the conditions that w remain finite as x → ∞ (yielding C1 = 0 and C2 = 0) as well as,

MO = −D⋅d2 wdx2|x = 0	and	QO = −D⋅d3 wdx3|x = 0

A variation in this solution is presented in the next section.

CONCENTRATED, EQUALLY SPACED MOMENTS ALONG CYLINDER

The cylindrical thin-walled shell configuration considered in this section has the same homogeneous solution given above.  That is, in dimensionless form,

β⋅L⋅wL = β⋅eβ⋅x⋅C1⋅cos β⋅x) + C2⋅sin (β⋅x + β⋅e−β⋅x⋅C3⋅cos β⋅x + C4⋅sin β⋅x

d wdx = β⋅eβ⋅x⋅C1⋅(cos (β⋅x) − sin (β⋅x)) + C2⋅(cos (β⋅x) + sin (β⋅x))  + β⋅e−β⋅x⋅-C3⋅(cos (β⋅x) + sin (β⋅x)) + C4⋅(cos (β⋅x) − sin (β⋅x))

1β⋅d2 wd x2 = 2⋅β⋅eβ⋅x⋅−C1⋅sin (β⋅x) + C2⋅cos (β⋅x) + 2⋅β⋅e−β⋅x⋅C3⋅sin (β⋅x) − C4⋅cos (β⋅x)

1β2⋅d3 wdx3 = 2⋅β⋅eβ⋅x⋅−C1⋅(sin (β⋅x)+ cos (β⋅x)) + C2⋅(cos (β⋅x) − sin (β⋅x)) + 2⋅β⋅e−β⋅x⋅C3⋅(cos (β⋅x) − sin (β⋅x)) + C4⋅(sin (β⋅x)+ cos (β⋅x))

The only external loading for this problem is a series of moment/(edge length) loadings, MO, that are equally spaced along the infinitely long cylinder. The axial spacing of the loadings is L.  Each external moment loading is a constant moment per unit edge length around the circumference of the cylinder and it is concentrated at a fixed axial position.  Since the loading pattern extends for the entire infinite length, the solution must be periodic.  If the solution is found between two adjacent loadings, it will be the same for the region between all adjacent loadings.  Therefore an appropriate problem to solve is sketched below.

x

QO

QO

MO/2

MO/2

a

wall thickness = h

QO

QO

MO/2

MO/2

w = w(x)

w = w(x)

L

A suitable set of boundary conditions for the above sketched problem are,

w(0) = w(L)

d wdx|x = 0 = d wdx|x = L

MO2⋅D = −d2 wdx2|x = 0

−MO2⋅D = −d2 wdx2|x = L

When these conditions are imposed on the homogenous solution, four linear algebraic equations for C1, C2, C3 and C4 are determined.  With the following definitions,

EP = eβ⋅L

EM = e-β⋅L

S = sin(β⋅L)

C = cos(β⋅L)

CMS = cos(β⋅L) − sin(β⋅L)

CPS = cos(β⋅L) + sin(β⋅L)

the equations for C1, C2, C3 and C4 are,

j = 14AIJ⋅CJ = BI

where,

A11 = −1 + EP⋅C

A12 = EP⋅S

A13 = −1 + EM⋅C

A14 = EM⋅S

B1 = 0

A21 = −1 + EP⋅CMS

A22 = −1 + EP⋅CPS

A23 = +1 − EM⋅CPS

A24 = −1 + EM⋅CMS

B2 = 0

A31 = 0

A32 = −2

A33 = 0

A34 = 2

B3 = MO2⋅β2⋅L

A41 = −2⋅EP⋅S

A42 = 2⋅EP⋅C

A43 = 2⋅EM⋅S

A44 = −2⋅EM⋅C

B4 = −MO2⋅β2⋅L

In order to demonstrate a few characteristics of this solution, a digital computer program was prepared.  The input data required for this program are,

BL = β⋅L 	and		DMO = MOβ⋅D

The figures presented below are for illustrative purposes.  The first two figures give results for the specific case of BL = 15.0 and DMO = 1.0.  These figures show that the axial separation of the moment loadings is sufficiently great so that the influence of the axial separation on the solution is minimal.

The two figures below are for the case of BL = 1.0 and DMO = 1.0.   They show that the axial separation of the moment loadings is sufficiently close so that the influence of the axial separation on the solution is significant.

The last three figures are for the case of DMO = 1 and BL varied.  Maximum dimensionless values for the displacement, the slope and the shear force per unit length are shown.

APPENDIX 2,  USEFUL ELASTIC, CANTILEVER, END LOADED BEAM SOLUTION INCLUDING SHEAR DEFLECTION

BACKGROUND

Timoshenko’s Elements of Strength of Materials presents the elementary beam theory including bend and shear deflection that is employed in this work.  Let x be the coordinate along the, originally straight, beam axis and y(x) be the lateral deflection of the axis when subjected to shear force loading, V(x), and moment loading, M(x).  The governing equation determining y(x) is,

d2 y(x)dx2 = M(x)EI(x) + kG⋅d dxV(x)A(x)

where,

EI(x)	= beam elastic bending stiffness, a function of x, 0 < x < LO

G	= elastic shear modulus

A(x)	= cross-sectional area of beam, a function of x, 0 < x < LO

k	= (shear stress on beam axis) / (mean shear stress on section) , not a function of x

The boundary conditions for a “built-in” condition at end x= 0 are,

y(0) = 0

d y(x)dx|x = 0 = −k⋅V(0)G⋅A(0)

The boundary conditions at end x = LO with external shear load, VO, and external moment load, MO, are,

M(LO) = MO

V(LO) = VO

Note that when the parameter k is set to zero the usual equations for beam theory without shear deflections are recovered.

DESCRIPTION AND SOLUTION FOR TAPERED CANTILEVER BEAM

The above figure defines the problem under consideration in this section.  The moment, M(x), and shear force, V(x),  in the region 0 > x > LO are,

M(x) = MO + VO⋅xo⋅LOxo − xxo

V(x) = VO

The bending stiffness, EI(x), and area, A(x), are,

EI(x) = 112⋅E⋅to⋅ho3⋅1 − xxo3

Ax = to⋅ho⋅1 − xxo

Introduce the following dimensionless parameters,

ξ = xxo	ψ(ξ) =yξxo	       ψ(ξ)' = d ψ(ξ)dξ	     ψ(ξ)'' = d2 ψ(ξ)dξ2

A1 = MO⋅xo + VO⋅xo⋅LOEI0 	A2 = VO⋅xo2EI0	A3 = k⋅VOG⋅ho⋅TO

The governing equation, given above, becomes,

ψξ '' = A1⋅11−ξ3 − A2⋅ξ1−ξ3 + A3⋅11−ξ2

with the boundary conditions,

ψ0 = 0	and	  ψ0 ' = A3

After integration and application of the boundary conditions the solutions for (and ’ are,

ψξ = yξxo = 12⋅A1⋅11−ξ − ξ − 1 − 12⋅A2⋅11−ξ + 2⋅ln1−ξ+ξ−1 − A3⋅ln1−ξ

and

ψξ ' = 1xo⋅∂ yξ∂ ξ = 12⋅A1⋅11−ξ2 − 1 − 12⋅A2⋅11−ξ2 − 21−ξ + 1 + A3⋅11−ξ

ILLUSTRATIVE EXAMPLE

Program TOOTH was prepared to implement the solution in the previous section.  The input data required are,

MO = END MOMENT, in-lbf

VO = END SHEAR FORCE, lbf

xo   = MAXIMUM POSSIBLE LENGTH, in

LO = LENGTH OF BEAM, in

TO = CONSTANT BEAM THICKNESS, in

ho = BEAM HEIGHT AT X = 0, in

EP = YOUNG'S MODULUS OF ELASTICITY, psi

NU = POISSON'S RATIO

As an illustrative example, the following input values were chosen,

MO =  1.

VO = 1.

xo = 4

LO = 2

TO = 1

ho = 1

EP = 30.E6

NU = 0.3

The output file for this case gives,

E	= PLANE STRAIN ELASTIC MODULUS 	= 32.97E6 psi

G	= SHEAR ELASTIC MODULUS		= 11.54E6 psi

EI(0)	= BENDING STIFFNESS AT x = 0		= 27.47E5 lbf-sq.in

A1	= 0.4368E-5

A2	= 0.5824E-5

A3	= 0.1300E-6

The following three figures graphically show the values ’ and Mξ⋅xoEIξ, respectively as functions of .

For the next two figures the loads are altered while the configuration is the same as the above three figures.  Each of the two following figures give ’ and Mξ⋅xoEIξ as a function of .  The loading for each figure are given on the figure.

APPENDIX 3,  A ROUGH APPROXIMATION FOR THE THREAD PROBLEM

INTRODUCTION

This appendix gives the development of a much simplified mechanical model for estimating the maximum load variation on teeth in a threaded connection.  In this model all of the individual teeth are replaced by an elastic shear layer between the outer and inner sections of tubing.  The elastic shear layer adheres to each of the tubes.  In addition, the simplified model does not account for the different mean radii of the inner and outer sections of tubing.  Couplings used with production tubing should have very low leakage rates.  This is often implemented through having a shoulder in the coupling and by pressure loss in the sealing fluid through the threads.  The shoulder seal is not considered here.  If one assumes that tooth distortions owing to tooth loading increases leakage along the threads, then it is desirable to have the teeth in the central section of the threads have relatively low loads compared to teeth near the ends of the threaded section.  In any event, increases in the length of the threaded section would be expected to reduce the sealing fluid leakage rate.  The difference between the radii of the inner and outer tubes is neglected in this model.  The elastic shear layer of the model considered here will have a shear stress distribution that, roughly, resembles the tooth load distribution.  The calculations for this simplified model are less complex and the associated computer program can help make preliminary design decisions concerning the required number of threads.

DESCRIPTION OF SIMPLIFIED MODEL INCLUDING GOVERNING EQUATIONS

The most significant difference between this model and the model described in the main part of this report is that the individual teeth are replaced by an elastic shear layer.  That is, the inner and outer tubes are without teeth and the radial gap between the tubes is filled with an elastic material that adheres to the contacting tube surfaces.  The length of the elastic layer equals the region that formerly contained teeth.

The sketch and nomenclature below help describe the simplified model.

uT x		= axial displacement of top section in x direction, in

uBx	= axial displacement of bottom section in x direction, in

EPS		= axial strain at loaded end of top section

F		= axial applied force at loaded end of top section

L		= axial length of elastic layer, in

H		= depth of all members (H = 1.0 in for programs), in

ET ,EB	= top and bottom sections Young’s moduli, respectively, psi

AT ,AB	= top and bottom sections cross-sectional areas, respectively, sq.in

T		= elastic shear layer thickness, in

KC		= spring constant for elastic shear layer, psi

(layer shear stress at position x per unit circumference) = xKC⋅uTx − uBxT

Let the axial displacements from the unloaded positions (F = 0.) for the top and bottom sections be designated as uTx and uBx, respectively.  Additionally, let the axial stresses for the top and bottom sections be designated as σTx and σBx, respectively.  The mechanical governing equations for this problem are,

σTx,x⋅AT−τx⋅H = 0

σBx,x⋅AB+τx⋅H = 0

ET⋅uTx,x = σTx

EB⋅uBx,x = σBx

τx = KCT⋅uTx−uBx

where a comma followed by a subscripted variable implies differentiation with respect to that variable.  The first two of the above five equations express axial force equilibrium requirements for the top and bottom sections.  The third and fourth equations are elastic stress-strain relations for the top and bottom sections.  The fifth equation is the elastic (shear stress)-(shear strain) relation for the shear layer between the top and bottom sections.

Clearly, the above differential equations may be integrated to obtain a solution.  For this study the differential equations are replaced by corresponding difference equations to enable a numerical method to solve the equations.  The reason for doing this is that if the current mechanical model is altered it will probably be straightforward to modify the numerical approach while this may not be true of the differential equation approach.  The numerical scheme is based on a collection of junctions interconnected by linear elastic members.  The layouts for the junctions and members is shown in the figure on Page 3 for N = 5.  The layout for other values of N may be deduced from this figure.  The equilibrium equations for this layout may be written in the matrix form as,

A⋅x = F

where,

A	= spring matrix, a (2⋅N − 1)×(2⋅N − 1) matrix containing spring constants

x(i)	= value of ith element of x , = ithjunction displacement, in

F(i)	= value of ith element of F , =  net force per unit circumference on ith junction,

lbf/in

As an illustrative example define,

KT	= outer cylinder axial spring constant per unit circumference, psi

KC	= annulus shear spring constant per unit circumference, psi

KB	= inner cylinder axial spring constants per unit circumference, psi

Then for the case of N = 4 the coefficient matrix, A , becomes,

A = KT −KT 0 0 0 0 0 −KT KC+2⋅KT 0 −KT 0 0 0 0 0 KC+2⋅KB −KC − KB 0 0 0 −KT −KC KC+2⋅KT 0 −KT 0 0 0 −KB 0 KC+2⋅KB −KC −KB 0 0 0 −KT KC KC+KT 0

In order to determine the displacements, x, the inverse of A denoted by A−1 must be found.  Then,

x = A−1⋅F

When the displacements have been found, the junction forces and member forces are easily calculated using the displacements and governing equations.

A FORTRAN computer program, Program SPRTESF, was prepared to implement the procedure described in this appendix.  As an illustrative example, set,

N	= 15

KT	= 13.49E6 psi

KC	= 2.547E6 psi

KB	= 13.17E6 psi

The output file for this input data is,

OUTPUT FILE FOR PROGRAM SPRTESF, FEBRUARY 18, 2017

UNITS OF FORCE ARE POUNDS/(CIRCUMFERENTIAL INCH)

FORCE ON OUTER CYLINDER AT LOADED END = 1.0 lbf/in

FORCE ON OUTER CYLINDER AT OTHER END = 0.0 lbf/in

INPUT DATA

N     = INDEX FOR STATIONS (1 TO 2*N-1)         15

KT    = TOP ROW SPRING CONSTANT, psi        .1349E+08

KC    = MIDDLE LAYER SPRING CONSTANT, psi    .2547E+07

KB    = BOTTOM ROW SPRING CONSTANT, psi      .1317E+08

MAT   = IF = 1, [A] AND [C] ARE PRINTED          2

VALUES FOR DISPLACEMENTS X(I) & JUNCTION FORCES F(I), inches & lbf/in

I, X(I), F(I) =  1      .9064E-07      .0000

I, X(I), F(I) =  2      .9064E-07      .5666E-07

I, X(I), F(I) =  3      .5840E-07      .7162E-08

I, X(I), F(I) =  4      .1078E-06     -.7893E-07

I, X(I), F(I) =  5      .1073E-06     -.5636E-07

I, X(I), F(I) =  6      .1342E-06      .3009E-06

I, X(I), F(I) =  7      .1509E-06      .5483E-07

I, X(I), F(I) =  8      .1657E-06     -.2605E-06

I, X(I), F(I) =  9      .1917E-06      .3057E-06

I, X(I), F(I) = 10      .2000E-06      .5912E-07

I, X(I), F(I) = 11      .2309E-06      .2094E-06

I, X(I), F(I) = 12      .2359E-06     -.3600E-06

I, X(I), F(I) = 13      .2691E-06     -.1843E-06

I, X(I), F(I) = 14      .2727E-06     -.5534E-06

I, X(I), F(I) = 15      .3066E-06      .2653E-06

I, X(I), F(I) = 16      .3102E-06     -.1828E-07

I, X(I), F(I) = 17      .3435E-06      .7716E-07

I, X(I), F(I) = 18      .3484E-06      .8707E-06

I, X(I), F(I) = 19      .3793E-06      .3218E-06

I, X(I), F(I) = 20      .3875E-06     -.9431E-06

I, X(I), F(I) = 21      .4136E-06     -.3062E-06

I, X(I), F(I) = 22      .4281E-06      .7021E-06

I, X(I), F(I) = 23      .4452E-06      .2656E-06

I, X(I), F(I) = 24      .4715E-06     -.1287E-05

I, X(I), F(I) = 25      .4716E-06      .1191E-05

I, X(I), F(I) = 26      .5198E-06      .2171E-06

I, X(I), F(I) = 27      .4887E-06     -.7761E-06

I, X(I), F(I) = 28      .5772E-06      .5492

I, X(I), F(I) = 29      .4887E-06      .0000

VALUES FOR MEMBER TENSILE FORCES (FM), lbf/in

I, FM =  1      .0000

I, FM =  2      .2309

I, FM =  3      .2309

I, FM =  4      .7691

I, FM =  5      .1257

I, FM =  6      .3566

I, FM =  7      .6434

I, FM =  8      .6858E-01

I, FM =  9      .4251

I, FM = 10      .5749

I, FM = 11      .3767E-01

I, FM = 12      .4628

I, FM = 13      .5372

I, FM = 14      .2115E-01

I, FM = 15      .4839

I, FM = 16      .5161

I, FM = 17      .1272E-01

I, FM = 18      .4967

I, FM = 19      .5033

I, FM = 20      .9159E-02

I, FM = 21      .5058

I, FM = 22      .4942

I, FM = 23      .9094E-02

I, FM = 24      .5149

I, FM = 25      .4851

I, FM = 26      .1251E-01

I, FM = 27      .5274

I, FM = 28      .4726

I, FM = 29      .2070E-01

I, FM = 30      .5481

I, FM = 31      .4519

I, FM = 32      .3680E-01

I, FM = 33      .5849

I, FM = 34      .4151

I, FM = 35      .6696E-01

I, FM = 36      .6519

I, FM = 37      .3481

I, FM = 38      .1227

I, FM = 39      .7746

I, FM = 40      .2254

I, FM = 41      .2254

I, FM = 42      .0000

The results in this output file are presented graphically in Figures 3 and 4 (Pages 8 and 9) of the main section of this report.

THIS IS FILE: COMPARE.DOC

E. Paul Cernocky, Gloria Valigura, Fred C. Scholibo, Jose Menchaca, Chris Burras and Larry Larson (1998), “A Standardized Approach to Finite Element Analysis of Casing-Tubing Connections to Establish Relative Sealing Performance as a Function of Design, Geometry, Machining Tolerance and Applied Loads,” Computational Mechanics, New Trends and Applications, S. Idelsohn, E. Onate and E. Dvorkin, (Eds.).

A. J. Hirshberg, M. C. Moyer and R. M. Rickenbach, (1988, September 1) “Surface-Casing Strain Capacity for North Slope Operations”, Society of Petroleum Engineers, SPE Drilling Engineering.

S. Timoshenko and S. Woinowsky-Krieger, Theory of Plates and Shells, Second Edition, McGraw-Hill Book Company, Inc., 1959