May 18, 2016

REMINDER OF AN OBSERVATION BY D. C. DRUCKER

by Paul R. Paslay, Texas P. E. # 44278, Manatee, Inc., F-4992

INTRODUCTION

The desire to have a visual representation for material, mathematical models has led to sketches representing the well known Maxwell and Voigt models for visco-elastic materials.  The classic treatise by H. Kolsky, Stress Waves in Solids, (Dover Publications, Inc., 1963) gives a discussion of these materials in Chapter 5.  The elastic and viscous contributions are based on linear models for each.  The formulation and its interpretation are properly considered for a corresponding continuous model of a real material.

Occasionally, an attempt is made to model a material conceptually by appealing to a sketch similar to those used in Stress Waves in Solids   but with elastic and/or viscous components that are nonlinear.  In Second-Order Effects in Elasticity, Plasticity and Fluid Dynamics, (The MacMillan Company, 1964) the article by D. C. Drucker, “Stress-Strain-Time Relations and Irreversible Thermodynamics,” (pages 331-351)  pointed out a consistency concern for constitutive equations derived from visualizations similar to the Maxwell and Voigt models that contain components of nonlinear materials.  If the model is to represent a generic element of the real material being modeled, then a second model may be constructed using multiple, identical elements of the model in both series and parallel arrays.  The usual rules of physics may then be used to determine the governing equation for the second model.  For current consistency considerations, the mathematical form for the second model must be the same as for the original model.

The following two sections demonstrate that,

1.) the Maxwell and Voigt models are consistent

2.) generally, a model containing simple nonlinear elements is not consistent as described above.

A

B

F

X

SERIES MODEL

FA = FB = F

XA + XB = X

XA + XB = X

.

.

.

PARALLEL MODEL

F

X

A

B

FA + FB = F

XA = XB = X

XA = XB = X

.

.

.

SUPERPOSED DOT = DERIVATIVE W.R.T. TIME

THE MAXWELL AND VOIGT LINEAR MODELS

The two sketches above show two general combinations for combining elements of a material.  These two combinations are adequate for obtaining the desired results in this study.  Consider first two Maxwell material elements in series.  Then the A and B elements each contain a viscous and an elastic sub element connected in series.  The constitutive equations for the elements are,

XA = CV⋅F + CE⋅FXB = CV⋅F + CE⋅F

with CV and CE being viscous and elastic constants, respectively, so that,

X = XA + XB = 2⋅CV⋅F + 2⋅CE⋅F

This equation is the constitutive equation for the two element model shown in the series model sketch.  It has the same mathematical form as each of its elements and thus satisfies the consistency condition mentioned above.

When the two Maxwell elements are connected in parallel as shown in the parallel model sketch,

2⋅X = XA + XB = CV⋅FA + FB + CE⋅FA + FB = CV⋅F + CE⋅F

This equation shows that the mathematical form for the parallel model using Maxwell elements also satisfies the consistency condition.

When the A and B elements are defined as Voigt elements the sub elements each contain a viscous and an elastic part that are connected in parallel.  The constitutive equations for the elements are,

FA = DE⋅X + DV⋅X

FB = DE⋅X + DV⋅X

With DE and DV being elastic and viscous constants, respectively, so that,

F = FA + FB = 2⋅DE⋅X + 2⋅DV⋅X

This equation shows that the mathematical form for the parallel model using Voigt elements satisfies the consistency condition.

Now consider the two Voigt model elements connected in series like the series model sketched above.  Then,

2⋅F = FA + FB = DE⋅XA + XB + DV⋅XA + XB = 2⋅DE⋅X + 2⋅DV⋅X

This equation shows that the mathematical form for the series model using Voigt elements satisfies the consistency condition.

The methods used above can be used to make models having many elements so the linear Maxwell and Voigt models satisfy the consistency condition pointed out by Drucker.

TWO NONLINEAR MODELS

Consider first two Maxwell material elements in series.  Then the A and B elements each contain a viscous and an elastic sub element connected in series.  The constitutive equations for the elements are taken to have the following nonlinear form,

XA = KV⋅F2 + KE⋅F2XB = KV⋅F2 + KE⋅F2

with KV and KE being viscous and elastic constants, respectively, so that,

X = XA + XB = 2⋅KV⋅F2 + 2⋅KE⋅F2

This equation is the constitutive equation for the two element model shown in the series model sketch.  It has the same mathematical form as each of its elements and thus satisfies the consistency condition

When the two Maxwell elements are connected in parallel as shown in the parallel model sketch,

2⋅X = XA + XB = KV⋅FA2 + FB2 + KE⋅FA2 + FB2  ≠ KV⋅FA + FB2 + KE⋅FA + FB2

This equation shows that the mathematical form for the parallel model using Maxwell elements does not satisfy the consistency condition.

When the A and B elements are defined as Voigt elements the sub elements each contain a viscous and an elastic part that are connected in parallel.  The constitutive equations for the elements are assumed to be,

FA = QE⋅X2 + QV⋅X2

FB = QE⋅X2 + QV⋅X2

With QE and QV being elastic and viscous constants, respectively, so that,

F = FA + FB = 2⋅QE⋅X2 + 2⋅QV⋅X2

This equation shows that the mathematical form for the parallel model using Voigt elements satisfies the consistency condition.

Now consider the two Voigt model elements connected in series like the series model sketched above.  Then,

2⋅F = FA + FB = QE⋅XA2 + XB2 + QV⋅XA2 + XB2 ≠ QE⋅XA + XB2 + QV⋅XA + XB2

This equation shows that the mathematical form for the series model using Voigt elements does not satisfy the consistency condition.

The two nonlinear models considered above both fail to satisfy the consistency condition being studied.

SUMMARY

It must be emphasized that nonlinear constitutive equations are legitimately employed in continuum mechanics and the consistency condition studied here is not a physical requirement.  The point of this reminder is that when a sketch is the basis for deriving a nonlinear constitutive equation, there is a gap in the logic of the derivation.

THIS IS FILE: DCD.doc